FENCING & HEDGE REQUIREMENTS

Fencing and/or hedges in residential zoned districts are permitted on all property lines. A Certificate of Zoning is required for all fences no matter the height or value of work. All fences over 6’ high require a building permit. In addition to the Certificate of Zoning, if replacing a damaged fence with like material and the fence area being replaced is not greater than 100 SF, no Certificate of Zoning or Building Permit is required. A copy of a site drawing is to be submitted with the Certificate of Zoning application. If a contractor is doing the work, he/she must be licensed with the City of Toledo as a Home Improvement and Remodeling Contractor and he/she will secure the Certificate of Zoning and the Building Permit if one is required.

Maximum fence heights from ground level to highest point of fence: Front yard – 3 ½ feet. Note that corner lots have two front yards. Side yard and Rear yard – 6 feet. The rear yard on a corner lot shall be at the opposite of the front yard with the parcel address.

Before proceeding with installation of fencing: (1) locate and ensure the accuracy of property lines. (2) notify neighbors as a courtesy. (3) check for deed restrictions and easements. (4) secure permit.

The finished side of all type fences (including chain link) must face the adjacent property, street or place, and shall not be constructed of used or discarded materials in a state of disrepair, including but not limited to: pallets, doors, tires, corrugated metal, tree trunks, or other similar items. Materials not specifically manufactured for fencing shall not be used for, or in the construction of a fence in a residential zoning district. per TMC Section 1108-0404(B).

CORNER LOTS HAVE

|  | REAR YARD |  |
| --- | --- | --- |
|  | House 1234 | SIDE YARD |

| REAR YARD |  |  |
| --- | --- | --- |
| SIDE YARD | House | SIDE YARD |

TWO FRONT YARDS	INTERIOR LOT

REAR YARD

6’ Maximum Height

SIDE YARD

\

6’ Maximum Height

3 ½’ Maximum Height